# 2. System Modeling and Problem Statement

## Notation / geometry (as on the figure)

- ed: lateral deviation (distance from vehicle CG to nearest point T on desired path).

- eφ=φd-φ: yaw angle error (desired yaw minus actual yaw).

- β: sideslip angle at CG (small).

- γ: yaw rate (paper’s sign convention — see remark below).

- vx: longitudinal velocity (assumed positive and large compared to lateral components).

- ρTσ: curvature of the desired trajectory at point T.

- Fyf,Fyr: front/rear lateral tire forces.

- Cf,Cr: cornering stiffnesses (front, rear).

- αf,αr: slip angles (front, rear).

- m: vehicle mass, Iz: yaw moment of inertia, lf,lr: distances from CG to front/rear axle.

- ΔMz: direct yaw moment produced by differences in longitudinal wheel forces (actuation by hub motors).

- Iw,wi,Ti,Fxi,Rt,ls — wheel inertia, wheel rotational speed, hub torque, longitudinal contact force, rolling radius, track-width related length, respectively.

2.1. System Modeling Considering Dynamic Characteristics of Steer-by-Wire

## 1) Kinematic lateral/yaw error dynamics → equation (1)

Goal: obtain

ed=vxeφ+vxβ+d1,eφ=γ+d2,                    ( SEQ Equation \* ARABIC 1)

Derivation of ed (lateral error rate)

Consider the vehicle velocity vector at the CG. The lateral component of CG velocity relative to the path tangent is approximately

vy≈vxβ since β small, vy=vxtanβ≈vxβ.

The path error angle between vehicle longitudinal axis and path tangent is eφ. The lateral motion of the CG relative to the path is dominated by the component vxsineφ+β. For small angles,

sineφ+β≈eφ+β.

Hence the lateral error derivative (ideal, no modeling error) is

ed≈vxeφ+β.

The paper adds d1 to represent modeling error and external disturbances, giving the first line in (1):

ed=vxeφ+vxβ+d1.

Derivation of eφ (yaw error rate)

By definition eφ=φd-φ. Differentiate:

eφ=φd-φ.

The desired yaw rate along the reference path equals vx ρTσ (curvature times forward speed). Denote the vehicle yaw rate by φ. Then

eφ=vxρTσ-φ.

Rewriting in the compact form eφ=γ+d2 simply collects the terms into a vehicle yaw-rate term γ (symbol for the measured yaw rate) and a disturbance term d2 representing the path curvature contribution (or its negative, depending on sign choice). In the paper they write d2=-ρTσvx so that

eφ=γ+d2,  d2=-ρTσvx,

which reproduces the compact equation (1). (This is only a book-keeping/sign-convention choice: the physical content is eφ=vxρT-φ.)

## 2) Yaw/lateral dynamics (vehicle planar dynamics) → equation (2)

β=1mvxFyf+Fyr+d3,γ=1IzlfFyf-lrFyr+1IzΔMz+d4.( SEQ Equation \* ARABIC 2)

Derivation of β (sideslip dynamics)

Start from lateral force balance in the vehicle body frame (standard bicycle / small-angle derivation). The lateral equation of motion about CG is

mvy+vxφ=Fyf+Fyr+(disturbance).

Using vy=vxβ (small β) gives vy=vxβ (neglecting higher order terms if vx taken as slowly varying). Substitute:

mvxβ+vxφ=Fyf+Fyr+(dynamics error).

Divide by mvx:

β+φ=Fyf+Fyrmvx+d,

where d collects unmodeled terms. If we adopt the paper’s grouping and sign conventions (they incorporate -φ into the symbol γ or into the disturbance d3), the dynamics are written compactly as

β=1mvxFyf+Fyr+d3.

(Again d3 captures the ±φ piece depending on the sign convention, plus unmodeled dynamics.)

Derivation of γ (yaw acceleration / moment balance)

Yaw moment equilibrium about CG:

Izφ=lfFyf-lrFyr+ΔMz+(disturbance terms).

It denotes γ as the yaw rate (or a signed version of it) and lumps disturbances into d4, then rearranging gives

γ=1IzlfFyf-lrFyr+1IzΔMz+d4.

Physical meaning: the first equation says lateral acceleration (through β) comes from lateral tire forces; the second says yaw acceleration comes from the moment arm difference of front/rear lateral forces plus any direct yaw moment ΔMz created by differential wheel longitudinal forces.

## 3) Direct yaw moment from differential longitudinal wheel forces → equation (3)

ΔMz=i=14-1iFxils2.                ( SEQ Equation \* ARABIC 3)

Derivation / explanation

- Fxi is the longitudinal force at wheel i (signed: forward positive).

- The wheels form two sides (left / right). A left–right imbalance in the longitudinal forces produces a net yaw moment about the CG.

- If ls denotes the track width (distance between left and right wheel centers), then each off-center longitudinal force produces a moment Fxi×ls/2 with sign depending on wheel side.

- Summing across all four wheels and applying alternating sign yields eq. (3):

ΔMz=i=14-1iFxi ls2.

This compactly encodes the net yaw torque produced by the hub motors through unequal longitudinal forces.

## 4) Wheel rotational dynamics → equation (4)

Iwwi=Ti-FxiRt.              ( SEQ Equation \* ARABIC 4)

Derivation / meaning

- For wheel i: rotational dynamics are Iwwi=torque applied by hub motor-torque due to tire longitudinal force.

- The torque resisting wheel rotation from the ground contact is FxiRt (longitudinal force times rolling radius).

- Denote motor torque by Ti. Then

Iwwi=Ti-FxiRt.

This is the standard single-wheel rotational model.

## 5) Lateral tire force linearization → equation (5)

Fyf=μCfαf,  Fyr=μCrαr.

Derivation / assumptions

- For small slip angles α the lateral tire force is often approximated linearly as Fy≈Cα α, where Cα is the cornering stiffness.

- The paper includes a multiplicative factor μ (road-adhesion coefficient) to account for friction scaling:

Fyf=μCfαf, Fyr=μCrαr.           ( SEQ Equation \* ARABIC 5)

- This is a first-order (linear) tire model valid for small slips and when a single scalar friction µ scales the maximum lateral force.

## 6) Slip-angle geometry → equation (6)

αf=β+lfvxγ-δf,  αr=β-lrvxγ.

Derivation

- The slip angle at the front axle equals the angle between the wheel plane and the velocity vector at the front axle. For small angles:

  - lateral velocity at front axle ≈ vxβ+lfφ (the yaw component at the front axle adds lfφ).

  - dividing by forward speed vx gives the effective angle contribution β+lf/vxφ.

  - subtract the steering angle δf (wheel steering reduces slip) to get αf.

- Doing the same at the rear (no steering) gives αr.

- we get

αf=β+lfvxγ-δf,  αr=β-lrvxγ.                 ( SEQ Equation \* ARABIC 6)

# 7) Steering actuator dynamics — derive equation (7)

Starting point / physical idea

- The steering assembly (steer-by-wire) has a rotational inertia Jw, viscous damping bw, a motor that produces torque proportional to motor current im, and it is loaded by the lateral tire force at the front wheel which produces a moment around the steering axis because the lateral force acts at a point offset from the steering axis. 

Torque balance around the steering axis (scalar)

Write rotational dynamics for the steering subsystem:

Jwδf+bwδf=Mmotor+Mtire+Mdist,

were

- Mmotor is motor torque applied to steering,

- Mtire is torque produced by the lateral tire force about the steering axis,

- Mdist lumps modeling errors (paper calls this d5).

Motor torque model

The motor torque delivered to the steering column is

Mmotor=ηmrskm im,

where ηm is motor efficiency, rs is steering ratio, km is motor constant and im motor current.

Tire-generated moment

- The lateral force at the front wheel is Fyf=μCfαf (eq. (5)).

- That lateral force acts at an effective lever arm equal to the sum of the aerodynamic offset lp and the mechanical offset lm, so the moment about the steering axis due to the lateral force is

Mtire=-lp+lm Fyf.

(Sign: chosen so that a positive lateral force producing a restoring moment appears with a minus sign in the steering dynamics as in the paper; the paper’s final sign convention produces the form in (7).)

Now substitute the linear tire model and slip-angle

Use Fyf=μCfαf and αf=β+lfvxγ-δf (eqs. (5) – (6)). Substitute:

Mtire=-lp+lm μCf​β+lfvxγ-δf.

Collect terms and divide by Jw

Plug motor & tire moments into the torque balance and divide by Jw:

δf+bwJwδf=ηmrskmJwim-lp+lm μCfJwβ+lfvxγ-δf+d5Jw.

Rearrange to isolate δf:

δf=-μCflp+lmJwβ-μCflflp+lmJwvxγ+μCflp+lmJwδf-bwJwδf+ηmrskmJwim+d5Jw.

This matches the eq. (7) (they include d5 as the lumped modeling error; the paper writes d5 directly on the right — we can keep d5Jw or absorb 1/Jw into the definition of d5; the paper denotes it simply d5).

Boxed final form (eq. (7))

δf=-μCflp+lmJw β-μCflflp+lmJwvx γ+μCflp+lmJw δf-bwJw δf+ηmrskmJw im+d5.            ( SEQ Equation \* ARABIC 7)

# 2.2. Polytopic Model Establishment for System Uncertainty

# 8) Parameter uncertainty parameterization — equation (8)

The paper models several uncertain scalar parameters as nominal + bounded time-varying perturbation (multiplicative form converted to additive on those scalars). The parameters and their decompositions (paper eq. (8)):

μt=μn+Δμ N1t,Cft=Cfn+ΔCf N2t,Crt=Crn+ΔCr N3t,1mt=1mn+1Δm N4t,1Izt=1Izn+1ΔIz N5t,                         ( SEQ Equation \* ARABIC 8)

with each scalar uncertainty multiplier Nit satisfying Nit≤1.

Nominal and half-range definitions

Defining the nominal value as the midpoint (or an equivalent harmonic form for reciprocals) and the uncertainty half-range as half the difference between max and min. Specifically (paper definitions):

- For μ, Cf, Cr:

- μn=12μmax+μmin, Δμ=12μmax-μmin, Cfn=12Cfmax+Cfmin, ΔCf=12Cfmax-Cfmin,

- and likewise, for Cr.

- For m and Iz the paper uses expressions involving harmonic means (since the uncertainty is presented for reciprocals 1/m and 1/Iz). The paper gives

- mn=2mminmmaxmmin+mmax,  Δm=2mminmmaxmmax-mmin, Izn=2IzminIzmaxIzmin+Izmax,  ΔIz=2IzminIzmaxIzmax-Izmin.

(These forms are taken exactly from the paper — they parameterize the reciprocals so that 1/m=1/mn+1/ΔmN4t holds with N4≤1.)

Interpretation

- This decomposition isolates the uncertain scalars into bounded, time-varying multipliers Nit with known magnitude bound (≤1), which is convenient to express the uncertain part of the system matrices as a factorized matrix.

# 9) Scheduling parameters (eq. (9)) — longitudinal velocity treated as time-varying

It treats longitudinal speed vx as a time-varying scheduling variable and selects three scheduling functions:

ρ1=vx,  ρ2=1vx,  ρ3=1vx2.                  ( SEQ Equation \* ARABIC 9)

Each ρi ranges over a known interval (lower/upper bounds determined from expected speed range). The paper writes:

ρ1=vx∈ vx, vx , ρ2=1/vx∈ 1/vx, 1/vx , ρ3=1/vx2∈ 1/vx2, 1/vx2 .

This choice is convenient because many coefficients in the linearized matrices involve factors vx, 1/vx or 1/vx2; representing them as independent scheduling parameters lets the model be expressed as an LPV (linear parameter varying) system and then embedded into a polytopic (convex hull) model over the vertex combinations of the three parameters.

# 10) State-space LPV / polytopic model — equation (10) and matrices

State / input / disturbance definitions

Collecting the plant states and inputs as

x=edeφβγδfδfT,  u=ΔMzim,  dt=d1d2d3d4d5T.

Using the linearized dynamic relations (including the steering dynamics derived above and the earlier lateral/yaw dynamics), the system can be written in LPV form:

xt=Aρxt+Bρut+Bddt,

and split into nominal + uncertain parts:

xt=Anρ+ΔAρxt+Bn+ΔBut+dt.               ( SEQ Equation \* ARABIC 10)

This is equation (10) in the paper.

How the physical terms map into Anρ, Bn, Bd

- Rows 1–2: the kinematic error dynamics (eq. (1)):

  - row 1 gives ed=vxeφ+vxβ+d1 → entries in row 1: columns for eφ and β equal ρ1 (since ρ1=vx).

  - row 2 gives eφ=γ+d2 → entry in row 2 column for γ is 1.

- Rows 3–4: lateral/yaw dynamics (substitute Fyf=μCfαf, Fyr=μCrαr and slip angles). After substitution and arranging, you get coefficients proportional to combinations of μCf, μCr, lf,lr, m, Iz and factors 1/vx or 1/vx2, which are represented using ρ2,ρ3 (see the explicit matrix in the paper). For example (paper notation using cf=μnCfn, cr=μnCrn for nominal values):

  - The β-coefficient in β is cf+crmn ρ2.

  - The γ-coefficient contains a ρ3 factor (from 1/vx2).

  - The input ΔMz appears in γ scaled by 1/Izn.
(See the explicit shown Anρ,Bn,Bd block in the paper for every entry.)

- Row 5–6: steering dynamics:

  - row 5 is δf=δf → a standard integrator row (0 ... 0 0 1).

  - row 6 comes directly from the steering dynamics derived in (7) and produces entries in the matrix depending on μCf and terms with a ρ2=1/vx factor for the γ term, as well as the motor current im input in Bn with coefficient ηmrskm/Jw.

Explicit nominal matrices (paper gives the numeric symbolic block)

The paper lists Anρ,Bn,Bd explicitly (I reproduce the structure — full matrix is in the paper):

Anρ=0ρ1ρ100000010000cf+crmnρ2cflf-crlrmnρ3-cfmnρ2000cflf-crlrIzncflf2+crlr2Iznρ2-cflfIzn000000100-cflp+lmJw-cflflp+lmJwρ2cflp+lmJw-bwJw
, Bn=0000001Izn0000ηmrskmJw,  Bd=I5×501×5T

Here the paper defines cf=μnCfn and cr=μnCrn.

Uncertain parts and factorization

- The uncertain parts ΔAρ and ΔB (due to Δμ,ΔCf,ΔCr, variations in 1/m and 1/Iz) are grouped and written in the factorized form

ΔAρ ΔB=H Λ L1ρ L2,                            ( SEQ Equation \* ARABIC 11)

with Λ diagonal consisting of a few repeated uncertain multipliers Nit, and specific matrices H,L1ρ,L2. This factorization is used later to apply the small-gain / S-procedure style LMI (Lemma 1) when deriving robust LMIs.

Polytopic embedding

Because the scheduling vector ρ=ρ1,ρ2,ρ3 ranges over a box (product of intervals), the LPV system can be embedded into a polytopic model with 8 vertices (all combinations of lower/upper values for each scalar). The paper writes this as

xt=i=18αiρ,tAni+ΔAix+Bni+ΔBiu+dt,                 ( SEQ Equation \* ARABIC 12)

with barycentric weights αiρ,t≥0, i​αi=1, and explicit αiρ,t formulas given in (13). This is a standard convex decomposition used to reduce synthesis to a finite set of vertex conditions.

x=edeφβγδfδf,  u=ΔMzim,  d=d1d2d3d4d5.

I’ll define the nominal composite cornering stiffnesses used by the paper:

cf≡μnCfn,  cr≡μnCrn,

and nominal reciprocals 1/mn, 1/Izn etc. Also recall the scheduling parameters

ρ1=vx, ρ2=1vx, ρ3=1vx2.

# i) — Intermediate substitutions (explicit)

Start from the linear tire models and slip-angle geometry:

Fyf=μ Cf αf,  Fyr=μ Cr αr, αf=β+lfvxγ-δf,  αr=β-lrvxγ.

Take nominal (nom) values for multiplicative constants: use cf=μnCfn, cr=μnCrn. For the nominal expressions we substitute μ→μn, Cf→Cfn, Cr→Crn. Then

Fyf≈cfβ+lfvxγ-δf,Fyr≈crβ-lrvxγ.

Compute the combinations that appear in the lateral/yaw dynamics:

- Sum of lateral forces:

Fyf+Fyr=cfβ+lfvxγ-δf+crβ-lrvxγ=cf+crβ+cflf-crlrvxγ-cfδf.

- Moment arm combination (front minus rear moments about CG):

lfFyf-lrFyr=lfcfβ+lfvxγ-δf-lrcrβ-lrvxγ=lfcf-lrcrβ+lf2cf+lr2crvxγ-lfcfδf.

(These two algebraic results are the core building blocks for rows 3 and 4.)

# ii) — Row-by-row derivations

### Row 1 — kinematic lateral error

From eq. (1): ed=vxeφ+vxβ+d1.

Thus row-1 of Anρ is

row1=0ρ1ρ1000,

and the disturbance d1 enters through Bd (see B_d below). There is no input u term in row 1, so corresponding Bn row is zeros.

### Row 2 — yaw-angle error

From eq. (1): eφ=γ+d2.

So row-2 of Anρ is

row2=000100,

again, no direct input term.

### Row 3 — sideslip rate β

From eq. (2):

β=1m1vx Fyf+Fyr+d3.

Substitute the nominal expression for Fyf+Fyr and pull out the dependence on vx:

β=1mn⋅1vxcf+crβ+(cflf-crlrvx)γ-cfδf+d3=cf+crmn⏟call aββ ρ2 β + cflf-crlrmn⏟call aβγ ρ3 γ - cfmn⏟call aβδ ρ2 δf + d3.

So row-3 of Anρ (coefficient vector for states ed,eφ,β,γ,δf,δf) is

row3=0, 0, cf+crmn ρ2, cflf-crlrmn ρ3, -cfmn ρ2, 0.

No direct input u (so corresponding row of Bn is zeros). Disturbance d3 enters row 3 (see Bd).

### Row 4 — yaw-rate acceleration γ

From eq. (2):

γ=1IzlfFyf-lrFyr+1IzΔMz+d4.

Substitute the nominal expression for lfFyf-lrFyr:

γ=1Iznlfcf-lrcrβ+(lf2cf+lr2crvx)γ-lfcfδf+1IznΔMz+d4=lfcf-lrcrIzn⏟aγβ β + lf2cf+lr2crIzn⏟aγγ ρ2 γ - lfcfIzn⏟aγδ δf + 1Izn⏟bγ,ΔMzΔMz + d4.

So row-4 of Anρ is

row4=0, 0, lfcf-lrcrIzn, lf2cf+lr2crIzn ρ2, -lfcfIzn, 0.

And the input matrix Bn row-4 (for u=ΔMz imT) is

row4Bn=1Izn0.

The disturbance d4 enters row 4 (see Bd).

### Row 5 — steering angle integrator

By definition δf=δf so row-5 is the integrator that picks δf state:

row5=000001,

no inputs and no disturbance directly (paper uses zero in B_n row 5 and zero in B_d row 5).

### Row 6 — steering actuator acceleration δf

From eq. (7) (we derived in previous page):

δf=-μCflp+lmJw β-μCflflp+lmJwvx γ+μCflp+lmJw δf-bwJwδf+ηmrskmJw im+d5.

Using nominal cf=μnCfn we write each coefficient:

aδβ=-cflp+lmJw,aδγ=-cflflp+lmJw ρ2,aδδ=cflp+lmJw,aδδ=-bwJw,bδ,im=ηmrskmJw.

So row-6 of Anρ is

row6=0, 0, -cflp+lmJw, -cflflp+lmJwρ2, cflp+lmJw, -bwJw,

and row-6 of Bn (for u=ΔMz imT) is

row6Bn=0ηmrskmJw.

Disturbance d5 enters row 6 (see Bd).

# iii) — Assemble Anρ, Bn, Bd explicitly

Collecting all rows, the nominal state matrix Anρ (6×6) is:

Anρ=0ρ1ρ100000010000cf+crmnρ2cflf-crlrmnρ3-cfmnρ2000lfcf-lrcrIznlf2cf+lr2crIznρ2-lfcfIzn000000100-cflp+lmJw-cflflp+lmJwρ2cflp+lmJw-bwJw.

The nominal input matrix Bn (6×2) is:

Bn=0000001Izn0000ηmrskmJw.

Finally, the disturbance matrix Bd maps d=d1,…,d5T into the states. From the paper’s definitions (disturbances d1 through d5 appear in rows 1,2,3,4,6 respectively), we have:

Bd=100000100000100000100000000001.

(Equivalently, the state equations include +d1 in ed, +d2 in eφ, +d3 in β, +d4 in γ, and +d5 in δf.)

# 11) — Factorization (equation (11))

The uncertain parts of the plant are written in the compact factorized form

ΔAρΔB=H Λ L1ρL2,

with a diagonal uncertainty matrix Λ that contains (repeated) bounded scalar uncertainty multipliers.

# i) — The matrices H and Λ

Choosing a 3×3 diagonal Λ (same scalar repeated) and a tall H that selects which state/input rows are affected:

Λ=diag{N1, N1, N1},  N1≤1, H=001000000100000001T=000000100010000001.

Interpretation: H places the uncertain combinations into the β-equation row (state 3), the γ-equation row (state 4), and the δf row (state 6).

# ii) — The L1ρ and L2 blocks (explicit)

The paper gives the following explicit 3×6 block L1ρ (rows correspond to the 3 uncertain channels that H will map into states 3,4,6):

L1ρ=00ϕ1+ϕ2 ρ2ϕ1lf-ϕ2lr ρ3-ϕ1 ρ2000ϕ3lf-ϕ4lrϕ3lf2+ϕ4lr2 ρ2-ϕ3lf000-Δcf lp+lmJw-Δcflflp+lmJw ρ2Δcflp+lmJw0.

The uncertain part of the input matrix is encoded by

L2=001ΔIz000.

Putting these together with H and Λ produces the full ΔAρ ΔB.

# iii) Definitions of the composite uncertainty coefficients

The short definitions uses for the composite uncertain coefficients are:

- composite cornering-stiffness variations:

Δcf=μnΔCf+Δμ Cfn+Δμ ΔCf, Δcr=μnΔCr+Δμ Crn+Δμ ΔCr.

- the ϕ-coefficients (they combine cornering-stiffness uncertainty and 1/m, 1/Iz uncertainty) are

ϕ1=cf+ΔcfΔm+Δcfmn,  ϕ2=cr+ΔcrΔm+Δcrmn, ϕ3=cf+ΔcfΔIz+ΔcfIzn,  ϕ4=cr+ΔcrΔIz+ΔcrIzn.

Here cf=μnCfn, cr=μnCrn, and mn,Izn,Δm,ΔIz are the nominal / half-range reciprocal parameters. These ϕi collect how cornering-stiffness and mass/inertia reciprocal uncertainties jointly perturb the linearized A entries.

# iv) How to read this factorization (what each term changes)

- The first row of L1ρ (mapped by H into the β row) contains the coefficients that multiply the state vector ed,eφ,β,γ,δf,δf to produce the uncertain additive contributions to β. Concretely, uncertainties in cornering stiffness and reciprocal mass produce additive terms proportional to ϕ1+ϕ2ρ2 (for β), ϕ1lf-ϕ2lrρ3 (for γ), and -ϕ1ρ2 (for δf).

- The second row of L1ρ (mapped into the γ row) gives uncertain contributions to γ from the same parametric variations (with different moment-arm combinations). The second row also produces the uncertain additive that multiplies the input ΔMz via L2 (the 1/ΔIz entry in L2 together with H,Λ produces uncertainty in the ΔMz input channel mapped into γ).

- The third row of L1ρ (mapped into the steering-actuator acceleration row δf) encodes how uncertainty in the front cornering stiffness (Δcf) perturbs the steering dynamics coefficients (terms multiplying β,γ,δf in row 6). The entries contain Δcf/Jw and the same ρ2 dependence for the γ-term as in the nominal row.

Finally, remaining parts of the uncertainties that are not represented in this factorized structure are lumped into the disturbance vector and treated as part of the overall disturbance dt.

# v) Expand the factorization into ΔA and ΔB

The paper gives (eq. (11))

ΔAρ ΔB=H Λ L1ρ L2.

Dimensions / shapes (for clarity)

- H is 6×3.

- Λ=diag{N1,N1,N1} is 3×3 with scalar unknown multiplier(s) N1t, N1≤1.

- L1ρ is 3×6, L2 is 3×2.

- So, product is 6×6+2=6×8 which partitions as ΔA6×6 and ΔB6×2.

Because Λ is diagonal with the same scalar N1 on the diagonal, we can simplify algebraically:

Λ=N1I3 ⇒ HΛ=N1H.

Therefore

ΔAρ=N1 H L1ρ,  ΔB=N1 H L2.

Now write H and L1,L2 explicitly (copying the paper’s structure):

H=000000100010000001, L1ρ=00ϕ1+ϕ2ρ2ϕ1lf-ϕ2lrρ3-ϕ1ρ2000ϕ3lf-ϕ4lrϕ3lf2+ϕ4lr2ρ2-ϕ3lf000-Δcflp+lmJw-Δcflflp+lmJwρ2Δcflp+lmJw0, 
L2=001ΔIz000.

Multiply H by each row of L1 (with scalar N1) — because of H's structure the result simply places the 1st row of L1 into row 3 of ΔA, the 2nd row of L1 into row 4 of ΔA, and the 3rd row of L1 into row 6 of ΔA. All other rows of ΔA are zero. Same for ΔB: the 2nd row of L2 placed into row 4 of ΔB.

So, the explicit expanded matrices are:

ΔAρ (6×6): only rows 3,4,6 nonzero

ΔAρ = N100000000000000ϕ1+ϕ2ρ2ϕ1lf-ϕ2lrρ3-ϕ1ρ2000ϕ3lf-ϕ4lrϕ3lf2+ϕ4lr2ρ2-ϕ3lf000000000-Δcflp+lmJw-Δcflflp+lmJwρ2Δcflp+lmJw0.

ΔB (6×2): only row 4 may be nonzero

ΔB = N10000001ΔIz00000.

# vi) Derive Δcf, Δcr and the ϕi coefficients

These are used to collect parameter uncertainty algebraically. Start from uncertain scalar decompositions (paper eq. (8)):

μ=μn+Δμ N1t, Cf=Cfn+ΔCf N2t, Cr=Crn+ΔCr N3t.

The nominal composite cornering stiffnesses are

cf=μnCfn,  cr=μnCrn.

When both μ and Cf vary, the full perturbed front cornering stiffness is

μCf=μn+ΔμN1Cfn+ΔCfN2=μnCfn+μnΔCfN2+ΔμCfnN1+ΔμΔCfN1N2.

It lumps the time-varying scalars into a single uncertain multiplier (they reorder which Ni multiplies what in the factorization). For the amplitude (half-range) they define the deterministic scalar Δcf equal to the maximum additive deviation from nominal (including cross-term).

Δcf=μnΔCf+Δμ Cfn+Δμ ΔCf.

Likewise for the rear:

Δcr=μnΔCr+Δμ Crn+Δμ ΔCr.

(These Δcf,Δcr are deterministic half-range magnitudes — they multiply the bounded scalar(s) used in the factorization.)

Now the ϕ-terms

These collect how the cornering stiffness uncertainty and reciprocal mass / reciprocal inertia uncertainty combine to perturb entries in ΔA.

ϕ1=cf+ΔcfΔm+Δcfmn,  ϕ2=cr+ΔcrΔm+Δcrmn , ϕ3=cf+ΔcfΔIz+ΔcfIzn,  ϕ4=cr+ΔcrΔIz+ΔcrIzn.

How these arise?

- Entries in β (row 3) include terms like cf+cr/m⋅1/vx. When cf,cr and 1/m are uncertain, the additive perturbation to that product can be approximated / bounded by terms proportional to combinations of Δcf,Δcr,Δ1/m. If the paper uses the reciprocal-parameter decomposition

- 1m=1mn+1ΔmN4t,

- then the perturbation piece contributed to the coefficient multiplying β is approximately

- cf+crΔm⏟nominal part times 1/ΔmN4+Δcf+Δcrmn⏟stiffness half-range / nominal recipNsth

- The paper collects these into ϕ1+ϕ2ρ2 etc where ϕ1,ϕ2 are defined as above so that the factorization becomes a linear combination of known deterministic matrices times bounded scalars.

- Similarly for the yaw row (row 4) the terms scale with 1/Iz and the ϕ3,ϕ4 entries collect those combinations.

You can use these ϕi directly in L1ρ as given.

# viii) How L1ρ is evaluated at the 8 vertices: L1i (and L2i)

The LPV scheduling vector is ρ=ρ1,ρ2,ρ3T=vx, 1/vx, 1/vx2T. Each ρk lies in an interval ρk,ρk (paper uses ρk and ρk notation for lower/upper).

To form a polytopic model we evaluate the parameter-dependent matrices at the 8 corner combinations (vertices) of the box determined by the three intervals.

Define the 8 vertex parameter triples (paper’s notation — matching image):

ρ1,ρ2,ρ3, ρ1,ρ2,ρ3, ρ1,ρ2',ρ3, ρ1,ρ2,ρ3,ρ1,ρ2',ρ3, ρ1,ρ2,ρ3, ρ1,ρ2',ρ3, ρ1,ρ2',ρ3,

(Where paper uses shorthand ρ2' etc to denote the alternate end of the interval — see your page; the exact order is not crucial as long as you consistently pair αi with the same vertex.)

For each vertex i∈{1,…,8} compute

L1i=L1ρi,  L2i=L2.

So, the 8 matrices L1i are formed by substituting each vertex’s ρ2,ρ3 values into the symbolic expressions in L1ρ. Concretely, for vertex i you substitute ρ2=ρ2i, ρ3=ρ3i into the three rows of L1⋅.

Example: at vertex where ρ2=ρ2, ρ3=ρ3, the first row of L1i becomes

0, 0, ϕ1+ϕ2ρ2, ϕ1lf-ϕ2lrρ3, -ϕ1ρ2, 0 .

Repeat for all 8 vertices — the paper lists the mapping L11=L1ρ1,ρ2,ρ3, L12=L1ρ1,ρ2,ρ3, ..., L18=L1ρ1,ρ2',ρ3.

L2 is constant (no ρ dependence) so L2i=L2 for all i.

# ix) Barycentric coordinates αiρ,t and Δρ

The paper uses barycentric coordinates over the 3D parameter box to write any ρ as a convex combination of the 8 vertices. The weights are

αiρ,t=k=13ρk-ρkiΔρ,                               ( SEQ Equation \* ARABIC 13)

where ρki is the value of the k-th parameter at the opposite vertex coordinate used to define each αi in the paper’s particular pattern (the page shows the explicit absolute-differences product form for each αi). The paper writes them concretely as (copied style):

α1ρ,t=ρ1-ρ1 ρ2-ρ2 ρ3-ρ3Δρ,α2ρ,t=ρ1-ρ1 ρ2-ρ2 ρ3-ρ3Δρ,  ⋮α8ρ,t=ρ1-ρ1 ρ2-ρ2 ρ3-ρ3Δρ,

with normalization denominator

Δρ=ρ1-ρ1 ρ2-ρ2 ρ3-ρ3.

These αi satisfy αi≥0 and i=18αi=1. They are used to write the LPV system as the convex combination in eq. (12):

xt = i=18αiρ,tAni+ΔAixt+Bni+ΔBiut+dt. ( SEQ Equation \* ARABIC 14)

# x) Vertex mapping for Ani (explicit list)

The paper defines each vertex nominal Ani by substituting the vertex parameter triple into Anρ. Using their indexing:

An1=Anρ1,ρ2,ρ3,An2=Anρ1,ρ2,ρ3,An3=Anρ1,ρ2',ρ3,An4=Anρ1,ρ2,ρ3,An5=Anρ1,ρ2',ρ3,An6=Anρ1,ρ2,ρ3,An7=Anρ1,ρ2',ρ3,An8=Anρ1,ρ2',ρ3.

Same for L1i=L1ρi and L2i=L2. The input matrices Bni=Bn are constant for all vertices in the paper.

# 2.3 Control Problem Formulation

For the trajectory tracking control layer of the FWIA EV, the full state vector is

x=ed eφ β γ δf δfT.

Ideally, state-feedback control requires that all six states be measured accurately.
However, in practical FWIA EVs:

- ed and eφ can be measured using low-cost sensors (e.g., GPS + IMU, camera-based localization).

- The yaw rate γ can be measured directly using onboard gyroscopes or indirectly reconstructed from inertial sensors.

- The front steering angle δf and its rate δf are available from the steer-by-wire motor encoders.

- But the sideslip angle β is difficult to measure accurately and cheaply, because:

  - It requires expensive high-grade IMUs or GPS-RTK slip-angle estimation,

  - Or sophisticated nonlinear observers,

  - And is highly sensitive to noise and tire parameter variation.

Because β is difficult to measure with inexpensive sensors, a full state-feedback controller is not practical.

### ⇒ Therefore, the paper adopts a robust output-feedback control approach.

Only the following measurable states are used as output:

yt=C1xt,

Where,

C1=100000010000000100000010000001.

So, the measured outputs are:

yt=ed eφ γ δf δfT.

Meanwhile, for performance evaluation the paper treats all states as performance output:

zt=C2xt,  C2=I6×6.

# LPV Polytopic Output-Feedback Plant Model

The plant model including:

- Time-varying parameters ρ=ρ1,ρ2,ρ3T,

- Polytopic blending coefficients αiρ,t,

- Nominal + uncertain matrices at each vertex.

The closed-form plant model is:

xt=i=18αiρ,tAni+ΔAixt+Bni+ΔBiut+dt,

with measurement and evaluation outputs:

yt=C1xt,  zt=C2xt.

This is the final LPV/polytopic model on which the output-feedback H∞ controller will be designed.

# Control Objective (H∞ Performance Requirement)

The objective of the output-feedback controller is to ensure:

- Robust asymptotic stability under parameter uncertainty and disturbances.

- Guaranteed H∞ disturbance attenuation, i.e. the tracking error energy is bounded relative to the disturbance energy.

The H∞ performance inequality is:

0tzTτzτ dτ ≤ v20tdTτdτ dτ,                   ( SEQ Equation \* ARABIC 15)

where v>0 is the desired attenuation level.

# Control Input / Actuator Constraints

Both control inputs are:

- Steering motor current im,

- Direct yaw-moment generating torque difference ΔMz.

Let

ut=u1tu2t=ΔMztimt.

The actuators must satisfy the physical constraints:

ust≤umax,s,  s=1,2,                      ( SEQ Equation \* ARABIC 16)

where:

- umax,1 is the maximum allowable yaw-moment command (determined by motor torque limits and tire–road conditions),

- umax,2 is the maximum steering-motor current limit.

These constraints must be enforced in the LMI synthesis.

In summary:

The controller must simultaneously ensure:

- robustness,

- trajectory tracking performance,

- disturbance attenuation,

- and actuator safety limits.

# Why Dynamic Output-Feedback?

The paper explains that dynamic output-feedback is chosen because:

- Not all states (especially sideslip angle β) can be measured,

- Static output-feedback is too restrictive and often infeasible,

- Dynamic output-feedback introduces controller internal state xc allowing full LMI-based H∞ synthesis.

This is the motivation leading directly into the controller structure

### Formulation (Reproduced)

For trajectory tracking control of the FWIA EV, the full state vector

x=ed eφ β γ δf δfT

is ideally required for feedback. The states ed and eφ can be measured using inexpensive positioning equipment, while the yaw rate γ can be obtained from onboard inertial sensors. The steering angle δf and its rate δf are available from the steer-by-wire motor sensors. However, the sideslip angle β is difficult to measure accurately with low-cost sensors. Therefore, a state-feedback controller is impractical, and an output-feedback-based control scheme is more suitable for FWIA EVs.

Accordingly, the measurable states ed,eφ,γ,δf,δf are selected as the output

yt=C1xt,  C1=100000010000000100000010000001,

and all states are used as the evaluation output

zt=C2xt,  C2=I6×6.

Using the polytopic LPV model and the parameter-dependent matrices, the control-oriented FWIA EV model becomes

xt=i=18αiρ,tAni+ΔAixt+Bni+ΔBiut+dt,

with outputs yt and zt as defined above.

The control objective is to design a robust H∞ dynamic output-feedback controller such that the closed-loop system is robustly asymptotically stable under parameter uncertainty and external disturbance, and satisfies the H∞ disturbance attenuation condition

0tzTτzτ dτ≤v20tdTτdτ dτ,

where v>0 is the prescribed attenuation level.
Additionally, actuator safety requires the control inputs

ust≤umax,s,  s=1,2,

where umax,s reflects motor torque and road-friction limits.

# 3. Robust H∞ Output-Feedback Trajectory Tracking Control (Reproduced)

To enhance the trajectory tracking performance under parameter uncertainty and external disturbances, the study employs a robust H∞ dynamic output-feedback controller of the form:

xct=i=18j=18αiρ,tαjρ,t Acij xct+j=18αjρ,t Bcj yt,ut=j=18αjρ,t Ccj xct+Dc yt,                 ( SEQ Equation \* ARABIC 17)

where:

- xct∈R6 is the controller state,

- Acij,Bcj,Ccj (for i,j=1,…,8) and Dc are controller gain matrices with appropriate dimensions,

- αiρ,t are the previously defined barycentric weights of the polytopic LPV model.

# Closed-loop System Formulation

Define the augmented state vector

x=xxc,

and let the augmented input disturbance mapping be

B=I6×602×6.

Also define the extended uncertainty mapping:

H=H02×3,  C=C206×2.

The closed-loop system combining plant and controller dynamics becomes:

xt=i=18j=18αiρ,tαjρ,t Aij+HΛLijxt+B dt, zt=C xt,         ( SEQ Equation \* ARABIC 18)

where the augmented vertex matrices are

Aij=Ani+BniDcC1BniCcjBcjC1Acij,  Lij=L1i+L2iDcC1L2iCcj.

This structure captures:

- the plant uncertainty via H,Λ,L1i,L2i,

- the controller influence via Acij,Bcj,Ccj,Dc,

- and the polytopic dependence via αiαj.

# Lemma 1 (Uncertainty Elimination via S-Procedure)

Lemma 1 (from [11]):
Let Y=YT, and let H,L be matrices of compatible dimensions.
Then the inequality

Y+HΛL+LTΛTHT<0                         ( SEQ Equation \* ARABIC 19)

holds for all diagonal uncertainty matrices Λ satisfying ΛTΛ≤I,
if and only if there exists ε>0 such that

YεHLT*-εI0**-εI<0.                            ( SEQ Equation \* ARABIC 20)

This lemma is essential because it converts the nonlinear uncertain inequality (19) into a linear matrix inequality (LMI) in terms of ε.

# Theorem 1 — LMI Conditions for Robust H∞ Performance

Let v>0 be the desired H∞ attenuation level and ϑ>0 the allowable bound on the controller state energy.
Assume scalars εij,εji>0 and a symmetric positive-definite matrix P=PT>0.

If the following LMIs hold:

Y1ij+Y1ji<0,  1≤i≤j≤8,                           ( SEQ Equation \* ARABIC 21)
                                                             Y2sj<0,  s=1,2, j=1,…,8,                         ( SEQ Equation \* ARABIC 22)

then the closed-loop system (18) is robustly asymptotically stable for all dt=0, and satisfies the H∞ performance condition (15) for all disturbances.

## Definition of the LMI matrices

The matrices appearing in (21) and (22) are:

### (a) Performance LMIs

Y1ij=AijTP+PAijPBCTPHLijT*-I000**-v2I00***-εijI0****-εjiI.
                                         Y1ji=AjiTP+PAjiPBCTPHLjiT*-I000**-v2I00***-εjiI0****-εijI.

These LMIs enforce:

- robust stability,

- H∞ disturbance attenuation,

- uncertainty elimination via Lemma 1.

### (b) Actuator Saturation LMIs

Define

Khj=DcC1Ccj.

Then

Y2sj=-umax,s2PKhjT*-ϑ-1I<0,  s=1,2, j=1,…,8.

These ensure that the control inputs satisfy:

ust≤umax,s,  s=1,2.

# Concise Proof of Theorem 1

To analyze stability and H∞ performance of the closed-loop system (18), consider the Lyapunov function candidate

Vx=xTP x,  P=PT>0.                           ( SEQ Equation \* ARABIC 23)

Now compute the time derivative of Vx and add the H∞ performance term
zTtzt-v2dTtdt.
For any attenuation level v>0, we obtain

Vx+zTtzt-v2dTtdt=i=18j=18αiρ,tαjρ,t xTΘijx+2xTPBd+zTz-v2dTd=i=18j=18αiρ,tαjρ,t xdTY1ijxd.( SEQ Equation \* ARABIC 24)

Here:

Θij = AijTP+PAij+PHΛLij+LijTΛTHTP,

and the combined matrix Y1ij is

Y1ij=Θij+CTCPBBTP-v2I.

# Main Inequality

From (24), if

i=18j=18αiρ,tαjρ,t Y1ij<0,                            ( SEQ Equation \* ARABIC 25)

then
V+zTz-v2dTd<0,
which implies robust asymptotic stability when dt=0 and satisfaction of the H∞ inequality (15) for all disturbances.

# Vertex Decomposition

Using the identities

αiαj=αi2,i=j,αiαj,i<j,

the inequality (25) can be rewritten as

i=18αi2ρ,t Y1ii + i=17j=i+18αiρ,tαjρ,tY1ij+Y1ji<0.                    ( SEQ Equation \* ARABIC 26)

A sufficient condition for (26) is:

Y1ij+Y1ji<0,  1≤i≤j≤8.                             ( SEQ Equation \* ARABIC 27)

This means checking only 36 LMIs instead of continuously over ρ.

# Using Schur Complement

Applying Schur’s complement to (27) yields:

ΘijPBCT*-v2I0**-I+ΘjiPBCT*-v2I0**-I<0.            ( SEQ Equation \* ARABIC 28)

# Applying Lemma 1

Recall that

Θij=AijTP+PAij+PHΛLij+LijTΛTHTP.

Using Lemma 1, the inequality (28) is satisfied if there exist εij,εji>0 such that:

AijTP+PAijPBCTεijPHLijT*-v2I000**-I00***-εijI0****-εjiI+AjiTP+PAjiPBCTεjiPHLjiT*-v2I000**-I00***-εjiI0****-εijI<0. (29)

These are exactly the LMIs stated earlier in Theorem 1 as conditions (21) – (22).

By applying a congruence transformation with

diag{ I, I, I, εij+εji-1I, I},

inequality (29) becomes equivalent to (21).

Thus, the LMI conditions of Theorem 1 guarantee robust stability and the H∞ bound.

# Final H∞ Bound

From inequality (30):

Vx+zTz-v2dTd≤0,              ( SEQ Equation \* ARABIC 30)

integrating both sides from 0 to t yields:

Vt≤v20tdTτdτdτ+V0≤v2∥dt∥22+V0≤ϑ, ∀t≥0,      ( SEQ Equation \* ARABIC 31)

where

- ϑ:=v2dmax+V0,

- dmax:=maxt∥dt∥22.

This shows that the Lyapunov function remains bounded and ensures robust H∞ performance for all disturbances.

To verify the actuator constraint (16), consider the controller (17) rewritten compactly as

u=Khxt,

where

Kh:=DcC1 j=18αjρ,tCcj.

Define the transformed variable

x=P1/2x,

so that

xTtxt=xTtPxt≤ϑ,

as established earlier from the Lyapunov inequality.

Then the control input satisfies

ust2=Khxt22≤maxt≥0xTtKhTKhxt2=maxt≥0xTtP-12KhTKhP-12xt2               ( SEQ Equation \* ARABIC 32)

which gives the bound:

ust2≤ϑ λmax​P-1/2KhTKhP-1/2,  s=1,2,

where λmax⋅ denotes the largest eigenvalue of its argument.

# Actuator Constraint Satisfaction

To ensure the actuator constraint

ust≤umax,s,  s=1,2,

it is sufficient that

ϑP-1/2KhTKhP-1/2<umax,s2I.

Using Schur’s complement and the identity

j=18αjρ,t=1,

we obtain:

-umax,s2PKhjT*-ϑ-1I=j=18αjρ,t-umax,s2PKhjT*-ϑ-1I=j=18αjρ,t Y2sj<0.            ( SEQ Equation \* ARABIC 33)

Therefore, when the LMI condition (22) of Theorem 1 holds, constraint (16) is satisfied, completing the proof. □

# Mathematical Transformation for Controller Synthesis

To derive solvable LMIs for the controller matrices, the matrix P and its inverse are partitioned as:

P=RFFTW,  P-1=SEETV,            ( SEQ Equation \* ARABIC 34)

where:

- R and S are symmetric positive definite matrices,

- F and E are full-rank matrices,

- they satisfy

- EFT=I-SR.

Define the matrices:

Q1=SIET0,  Q2=IR0FT.              ( SEQ Equation \* ARABIC 35)

These matrices will be used to transform the nonlinear LMIs of Theorem 1 into the linear LMIs of Theorem 2, allowing controller matrices Acij,Bcj,Ccj,Dc to be solved explicitly.

### Detailed Prove of Theorem 1:

#### The Setup: System and Stability

We start with the augmented closed-loop system dynamics

We use the Lyapunov function:

We aim to satisfy the  performance criterion:

#### 2. Expanding the Time Derivative ()

First, calculate the time derivative of the Lyapunov function along the system trajectories.

Substitute  into the equation:

Expand the transpose terms (using ):

Distribute the multiplication:

Combine the scalar cross terms (since  and the result is a scalar, they are equal):

#### 3. Formulating the Matrix Inequality

Substitute the expanded  and  into the  condition :

Group the terms into a block matrix multiplied by the vector :

For this inequality to hold for all states, the matrix itself must be negative definite:

#### 4. Applying Schur Complement

The term  makes the inequality quadratic in . We use the Schur Complement to linearize it. The lemma states that  is equivalent to . (Here ).

Applying this to remove :

#### 5. Isolating the Uncertainty

Expand the term involving :

Substitute the uncertainty definition :

Now, rewrite the full matrix inequality as the sum of a Nominal Matrix () and the Uncertain Terms:

#### 6. Structuring for Lemma 1

We express the uncertainty in the form  to match Lemma 1.

Define column vector  and row vector :

Then the inequality becomes:

#### 7. Applying Lemma 1 (Theorem 1 Result)

Lemma 1 states that  is true if and only if there exists  such that:

Substituting our specific matrices (, , ) into this block structure results in the final  matrix inequality.

The Full Matrix:

# Theorem 2

Theorem 2.
Given the positive constants v and ϑ, suppose there exist symmetric positive definite matrices

R>0,  S>0,

positive scalars εij and εji, and matrices

Acij, Bcj, Ccj, Dc,

such that the following LMIs are satisfied:

Ξ1ij+Ξ1ji<0,  1≤i≤j≤8,                ( SEQ Equation \* ARABIC 36) 
Ξ2sj<0,  s=1,2,  j=1,…    ( SEQ Equation \* ARABIC 37)

then the closed-loop system (18) is asymptotically stable under dt=0, and satisfies the H∞ performance condition (15) for all disturbances dt.

# LMI Matrices Ξ1ij, Ξ1ji, and Ξ2sj

The matrices in (36)–(37) are defined as follows:

### (a) LMI for robust H∞ stability:

Ξ1ij=Gij+GijTAcij+KiISC2THSL1iT+CcjTL2iT*Jij+JijTRC2TRHL1iT+C1TDcTL2iT**-I000***-v2I00****-εijI0*****-εjiI,
                           Ξ1ji=Gji+GjiTAcji+KiISC2THSL1jT+CcjTL2iT*Jji+JjiTRC2TRHL1jT+C1TDcTL2iT**-I000***-v2I00****-εjiI0*****-εijI.

Here

Gij=AniS+BniCcj,  Ki=Ani+BniDcC1, Jij=RAni+BcjC1.

### (b) LMI for actuator constraints:

Ξ2sj=-umax,s2 S-umax,s2 ICcjTC1TDcT*-umax,s2RCcjTC1TDcT**-ϑ-1I0***-ϑ-1I.                      ( SEQ Equation \* ARABIC 38)

These LMIs ensure that the control inputs satisfy the magnitude constraints (16) at every vertex.

# Controller Reconstruction (Recovering Acij,Bcj,Ccj,Dc)

Once the LMIs of Theorem 2 are solved, the actual controller matrices for (17) are recovered using the following transformations:

Acij=F-1​Acij-RAni+BniDcC1SE-T-BcjC1SE-T-F-1RBniCcj,Bcj=F-1Bcj-RBniDc,Ccj=Ccj-DcC1SE-T,Dc=Dc.       ( SEQ Equation \* ARABIC 39)

These equations convert the transformed controller variables
Acij,Bcj,Ccj,Dc
into the actual controller matrices used in the output-feedback law (17).

Note that R,S,E,F come from the block-partitioned inverse of matrix P:

P=RFFTW,  P-1=SEETV,

with

EFT=I-SR.

# Concise Proof of Theorem 2

Proof.
To derive Theorem 2 from Theorem 1, perform a congruence transformation on inequality (21) using

diag{Q1, I, I, I, I},

and on inequality (22) using

diag{Q1, I}.

Under these transformations, the inequalities Y1ij/Y1ji and Y2sj become the following matrices:

Ξ1ij=Q1TAijTPQ1+Q1TPAijQ1Q1TPBQ1TCTQ1TPHQ1TLijT*-v2I000**-I00***-εij-1I0****-εjiI,Ξ1ji=Q1TAjiTPQ1+Q1TPAjiQ1Q1TPBQ1TCTQ1TPHQ1TLjiT*-v2I000**-I00***-εji-1I0****-εijI,Ξ2sj=-umax,s2Q1TPQ1Q1TKhjT*ϑ-1I.       ( SEQ Equation \* ARABIC 40)

These expressions now match the structure required by inequalities (36)–(37) in Theorem 2.

# Substitution Using Definitions of Q1 and Q2

From the definitions in (35), the matrices satisfy:

PQ1=Q2.

Using this identity, we compute the following intermediate results:

Q1TPAijQ1=Q2TAijQ1=AniS+BniCcjAni+BniDcC1AcijRAni+BcjC1,Q1TPB=Q2TI0=IR,Q1TCT=SC2TC2T,Q1TPH=Q2TH0=HRH,Q1TLijT=SETIET0L1iT+L2iDcC1TL2iCcjT=SL1iT+CcjTL2iTL1iT+C1TDcTL2iT,Q1TPQ1=SIRS,KhjQ1=DcC1CcjSIET0=CcjDcC1.         ( SEQ Equation \* ARABIC 41)

# Recovered Controller Variable Relations

To convert the transformed controller variables
Acij,Bcj,Ccj,Dc
to the original controller matrices for (17), the following identities hold:

Acij=RAni+BniDcC1S+FBcjC1S+RBniCcj+FAcijET,Bcj=RBniDc+FBcj,Ccj=DcC1S+CcjET,Dc=Dc,  i,j=1,…,8.                   ( SEQ Equation \* ARABIC 42)

Substituting the relations in (41) into (40) yields exactly the LMIs (38).
Similarly, inverting (42) produces the reconstruction formulas (39).
This completes the proof.

### Detailed Prove of Theorem 2:

#### 1. The BMI Obstacle

From Theorem 1, the robust stability condition:

Υij+Υji<0

where Υij contains the bilinear term PAij with:

Aij=Ani+BniDcC1BniCcjBcjC1Acj

This is a Bilinear Matrix Inequality (BMI) since P multiplies controller matrices, creating non-convex terms.

#### 2. Partitioning the Lyapunov Matrix

P=RFFTW, P-1=SEETV

with R,S∈Rn×n symmetric positive definite. From PP-1=I, the 1,1 block yields:

RS+FET=I

#### 3. Defining Transformation Matrices

Q1=SIET0, Q2=IR0FT

Verification (PQ1=Q2):

PQ1=RFFTWSIET0=RS+FETRFTS+WETFT=Q2

#### 4. Change of Variables (Linearization)

Dc=DcCcj=DcC1S+CcjETBcj=FBcj+RBniDcAcij=RAni+BniDcC1S+FBcjC1S+RBniCcjET+FAcjET

#### 5. Congruence Transformation Setup

Apply T=diagQ1,I,I,I,I to Υij<0. Then:

Q1TPAijQ1=Q2TAijQ1

#### 6. Algebraic Expansion (Explicit Linearization)

Π=Q2TAijQ1=I0RFAni+BniDcC1BniCcjBcjC1AcjSIET0

6.1 Top-Left Block (1,1):

Π11=Ani+BniDcC1S+BniCcjET=AniS+BniCcj

6.2 Bottom-Left Block (2,1):

Π21=RAni+BniDcC1S+FBcjC1S+RBniCcjET+FAcjET=Acij

6.3 Top-Right Block (1,2):

Π12=Ani+BniDcC1=Ani+BniDcC1

6.4 Bottom-Right Block (2,2):

Π22=RAni+RBniDc+FBcjC1=RAni+BcjC1

#### 7. Transforming Auxiliary Blocks

- Disturbance: Q1TPB=Q2TB=Bd;RBdT

- Output: CQ1=C2S C2

- Uncertainty: Q1TPH=H;RHT

- Bounds: LijQ1=SL1i+L2iCcj L1i+L2iDcC1

#### 8. Final LMI Assembly

Ξ1ij=Gij+GijTKi+AcijTISC2THSL1iT+CcjTL2iT*Jij+JijTRC2TRHL1iT+C1TDcTL2iT**-2ω2I000***-2I00****-2εij-1I0*****-2εijI

where Gij=AniS+BniCcj, Ki=Ani+BniDcC1, and Jij=RAni+BcjC1.

Final stability condition:

Ξ1ij+Ξ1ji<0, 1≤i≤j≤8

#### 9. Controller Reconstruction

Once the solver determines the linear variables:

- Find non-singular F,E using SVD such that FET=I-RS

- Invert the definitions:

Dc=DcCcj=Ccj-DcC1SET-1Bcj=F-1Bcj-RBniDcAcj=F-1Acij-RAni+BniDcC1S-FBcjC1S-RBniCcjETET-1